Debugging in Linux

Presents the Linux commands to monitor, inspect the system state and debugging methods in Linux

- Pre Requisite

  - Have developer package installed and built

- Content

1.	View system state with debugfs

2.	Know which Linux kernel peripheral driver are started at boot time

3.	Device tree in the VFS.

4.	Peripheral VFS information UART example  (tty in linux)

5.	Example of I2C sysfs access of Humidity&temp sensors connected to I2C5 with extension board

6.	VFS overview

7.	Find device driver source code behind a device entry in /dev.

8.	Linux Kernel log

9.	TF-A /U-boot log

10.	Framework debug log - DRM KMS, V4L2, GStreamer, Wayland Weston, Alsa

11.	Advanced Linux advanced traces

12.	Monitoring tool

13.	A7 step-by-step debugging with GDB (Gnu DeBugger)

13.1	Debug TF-A

13.2	Debug the Linux kernel

13.3	Debug U-boot

13.4	Debug an application

Color convention

Blue rectangle: commands on Linux Host

Open Linux Terminal Window and copy/paste into terminal window

Linux host command lines

Linux host termial window prompt

Ex: Open Hyperterminal with following command line from Linux host

minicom –D /dev/ttyACM0

Then copy/paste board command lines in Linux host terminal window

Pink rectangle: commands on the board

Type commands  in the hyper terminal window connected to the board

Board command lines

Target hyper terminal window prompt

Wiki user guide reference

Debugfs

How to use the kernel dynamic debug

How to find Linux kernel driver associated to a device

Trace and debug tools

STM32MP1 Platform trace and debug environment overview

Debugging tools

GDB

GDB commands

Gdbgui

Debugging the Linux kernel using the GDB

U-Boot-How to debug

TF-A-How to debug

OP-TEE-How to debug

STM32-CoPro-MPU plugin for SW4STM32

Serial/TTY overview

This documents refers some time to division “STM32MP1 handsn Trace and Debug.pptx”

about trace and debug that is located in

- here

or

- http://best.st.com/docshare/MCDAppli/Trainings/Documents/Trainings/Products/MP1/MP1%20hands%20on/STM32MP1%20hands%20on%20Trace%20and%20debug%20ext.pdf

- View system state with debugfs

The kernel provides Virtual files (Virtual file system (VFS) means in RAM initialized at boot, not a permanent storage in Flash

that gives information about the system configuration and device drivers state.

See the article:  [ Debugfs ]

- You can see information about the current setting of many STM32MP1 peripheral.

Under /sys/kernel/debug the files are filled by the linux peripheral drivers.

- Interesting configuration info on clock tree values, regulator values, gpios, standby or stop wake-up sources, peripheral registers values

If you want to know how the RCC clock are configured (by TF-A) look the TF-A device tree source or in the procfs, see article for further info [STM32MP15_clock_tree]

More info in chapter 6 of https://wiki.st.com/stm32mpu/index.php/Regulator_overview

More info in chap 5 of https://wiki.st.com/stm32mpu/wiki/Pinctrl_overview#How_to_debug

Read access to some registers of some peripheral like STPMIC1

- Know which Linux kernel peripheral driver are started at boot time

List of all the device drivers ( ie STM32MP1 peripheral driver) that are initialized

ie  whose driver is “probed” at kernel boot time . There 2 devices type: character and block devices (for storage)

cat /proc/devices

Example : if you want to know which STM32 Uart is started

ls -d /sys/devices/platform/soc/*.serial

- device tree in the VFS.

All the nodes declared in the device-tree, including the disabled ones are under: /proc/device-tree

All nodes of the STM32MP1 are under /proc/device-tree/soc

- At boot time, the kernel starts (probe) the peripherals whose field in the device tree is “Status=OKAY”.

- You can check if a device is configured to be started or not at boot time.

/proc/device-tree/soc/i2c@40012000/status contains “OKAY” means driver will be “probed’.

- Device tree “compatible” field tell the name of the drivers that can drive this IP.

/proc/device-tree/soc/i2c@40012000/compatible

- In the driver source code retrieve from this name (grep) the exact the driver source file.

“od” below is to open binary file
(see hand’son  lab1 slide 21 for more info.)

cd /proc/device-tree/soc/i2c@40012000 cat compatible cat status od -xi --endian=big i2c-scl-rising-time-ns

- Peripheral VFS information USART example (tty in linux)

https://wiki.st.com/stm32mpu/index.php/Serial_TTY_overview

To know

- which USART driver is started (probed) at boot time

- which STM32 USART is behind ttySTM0 or ttySTM1

To know how the UART is configured (speed, parity, …)

read the files in the /proc/device-tree

- Example of I2C sysfs access of Humidity&temp sensors connected to I2C5 with extension board

http://intranet.lme.st.com:8000/php-bin/mm_wiki/index.php/IKS01a2_with_STM32mp157c-dk2

- VFS overview

Short descriptions of a sysfs , procfs and tempfs

https://wiki.st.com/stm32mpu/index.php/Pseudo_filesystem#sysfs_.28.2Fsys.29_-_System_filesystem

- Find driver source code behind a device entry in /dev.

See article to see how to retrieve the driver source code from the device of /dev

Ex :  How to find the source code of the device driver of /dev/ttySTM0 (UART4) ?

[ How to find Linux kernel driver associated to a device ]

- Linux Kernel log

The Kernel log is the most first common way to get traces log when something is getting wrong
 in the kernel side.

All the kernel drivers/modules send trace messages in a kernel ring buffer that is accessible for reading with dmesg command. Also at early stage -before the ring buffer is initialized-, we can get also the early boot messages.

See complete information on dmesg

[ Dmesg and Linux kernel log ]

  - See the traces

Each message sent by the drivers/modules has a “log level”, pre-defined by the source driver code developer.

The messages with log level 0-6 are always present in the kernel ring

The debug messages (log level 7) are present in the ring buffer under condition: the driver/module has to be compiled with the macro DEBUG enabled or if the dynamic debug is enabled for a given driver/ module.

In parallel, these messages can be sent to the console (via the UART4 -on ST reference board).

The messages arriving on the console can be filtered according to the console log level filter (the are all in the ring buffer)

The  “console log level” is stored in /proc/sys/kernel/printk

“Current” console_log leve is 7

=> message 1-7 => the debug messages (loglevel 8) are not visible on the console.

If you want to get the debug messages into console at boot time, change the current console_log level

For that, modify the kernel command line in the extlinux.conf file

root=/dev/mmcblk0p5 rootwait rw console=ttyS3,115200 loglevel=8

If you want to get debug message into the console, change the current console log level:

dmesg –n 8 or echo 8 > /proc/sys/kernel/printk

If you want to clear the kernel ring buffer

dmesg –C

  - how to activate the debug messages from one kernel module/driver?

Debug trace activation can be at compilation time (define DEBUG) or at run time with the “kernel Dynamic debug” methode

kernel Dynamic debug

The debug message can be activated for a whole module, for file in the module if know in which file send a trace message, or for given message at a line in the file.

This is thanks to /sys/kernel/dynamic_debug/control VFS file.

Go further in article: [ How to use the kernel dynamic debug ]

Example:

Dynamic debug trace activation for a given file of Usb driver

echo 'file drivers/usb/core/hub.c +p' > /sys/kernel/debug/dynamic_debug/control

Plug the USB key, observe the “hub”, “usb” prefixed debug messages in the ring buffer with dmesg,

Note that they are not visible in the console.

dmesg

If you want to see , the “hub” “usb” debug messages in the console

dmesg –n 8

unplug, Usb key , then observe the “hub” “usb” debug messages are now coming into the console

Traces of the Uart driver

by sending “toto” characters from target (via ttySTM0 -Uart4) to the Linux host terminal console

echo "file drivers/tty/* +p" > /sys/kernel/debug/dynamic_debug/control dmesg -n 8 echo toto > /dev/ttySTM0

Go further info in article: Serial TTY overview

  - Debug the Kernel drivers: use dynamic debug but not only

Question is to how to activate the dynamic debug traces for a given kernel module/driver?

The picture below allows accessing to different Linux software frameworks which provide specific trace and debug information in their "How to trace and debug the framework" dedicated chapter.

Click below on desired IP to get the IP overview article then goes into “how to trace and debug the framework” to activate the traces

- ALSA overview

- Category:Arm architecture

- Bluetooth overview

- Clock overview

- CEC overview

- Linux remoteproc framework overview

- Resource manager for coprocessing

- Linux Mailbox framework overview

- Linux RPMsg framework overview

- Coprocessor management overview

- Hardware random overview

- Crypto API overview

- OP-TEE overview

- Linux kernel device tree

- Dmaengine overview

- DRM KMS overview

- Vivante GCNANO GPU overview

- I2C overview

- IIO overview

- Interrupt overview

- Overview of GPIO pins

- MMC overview

- MTD overview

- NVMEM overview

- CAN overview

- Ethernet overview

- WLAN overview

- Power overview

- Regulator overview

- Thermal overview

- Reset overview

- RTC overview

- Serial TTY overview

- SPI overview

- PWM overview

- USB overview

- V4L2 camera overview

- Watchdog overview

See wiki :

- STM32MP15 Linux kernel overview

- https://wiki.st.com/stm32mpu/index.php/Linux_tracing,_monitoring_and_debugging#Trace_and_debug_overview_per_Linux_software_frameworks

Example

- For SPI click on SPI overview goes

to 5.1.2 Dynamic trace you get the following instruction to activate the debug traces in kernel ring buffer with dmesg

echo "file spi* +p" > /sys/kernel/debug/dynamic_debug/control

if you want to see the message in the console

dmesg –n 8

- For USART Serial TTY overview

echo "file drivers/tty/* +p" > /sys/kernel/debug/dynamic_debug/control

  - Add debug trace in the kernel module or driver

Add in the source code

printk("GPIO example exit myName \n");

and recompile

you will see the trace with dmesg.

See the lab adding a trace in a kernel module in the doc Lab-DeveloperPackageVx.docx in here

“step 10 : STM32MP1 Developer Package Debug Linux kernel module with printk”

  - Additional traces for the kernel framework

Some framework in the kernel and some application can log interesting message in

/var/log/messages file.

cat /var/log/messages

- TF-A /U-boot log

These logs are not visible via dmesg as the Kernel is not started at this stage.

- TF-A log ; This give board, Soc revision , boot reason (wake up exit), DDR configuration.

See the traces in here

By default TF-A the debug release in the distribution we see INFO, NOTICE TF-A log level.

In TF-A code :

include/common/debug.h

#define LOG_LEVEL_NONE                               U(0)

#define LOG_LEVEL_ERROR                            U(10)

#define LOG_LEVEL_NOTICE                U(20)

#define LOG_LEVEL_WARNING                        U(30)

#define LOG_LEVEL_INFO                                U(40)

#define LOG_LEVEL_VERBOSE                        U(50)

tf-a-stm32mp-2.0-r0/arm-trusted-firmware-2.0/Makefile

# Use LOG_LEVEL_INFO by default for debug builds

LOG_LEVEL          :=         40

else

# Use LOG_LEVEL_NOTICE by default for release builds

LOG_LEVEL          :=         20

- U-boot

See the traces in here

- Framework debug log -

DRM KMS, V4L2, GStreamer, Wayland Weston, Alsa

Log activation and content is specific to the framework. See the page below.

- Display:  DRM KMS

https://wiki.st.com/stm32mpu/index.php/DRM_KMS_overview

There are chapters dedicated:

- Video playback:  GStreamer

No info on wiki

- Camera:  V4L2

https://wiki.st.com/stm32mpu/index.php/V4L2_camera_overview

- Windowing: Weston

Log can be activated

https://wiki.st.com/stm32mpu/index.php/How_to_debug_Weston

Activate the Weston log traces into the console

export WAYLAND_DEBUG=1 weston-terminal

Activation of the wayland terminal

Examples of appli : Weston-simple

https://wiki.st.com/stm32mpu/index.php/Wayland_Weston_overview

https://wiki.st.com/stm32mpu/index.php/How_to_stop_and_start_Weston

export WAYLAND_DEBUG=1 weston-simple-egl

Weston init traces into in a log file

cat /var/log/weston.log

- Audio : ASLA

https://wiki.st.com/stm32mpu/index.php/ALSA_overview

- Advanced Linux advanced traces

  - Ftrace to observe the function call in the kernel side

Ftrace show all the kernel functions that are called by an application.

- First, recompile the kernel and external module with Ftrace enabled

https://wiki.st.com/stm32mpu/index.php/Ftrace#Using_the_STM32MPU_Embedded_Software_distribution

bitbake virtual/kernel -c menuconfig

Kernel uImage is precompiled with Ftrace enabled in here , copie uImage into your VM desktop ( ~/Desktop )

Install on target uImage (kernel image)

| scp ~/Desktop/uImage root@192.168.7.2:/boot |  |
| --- | --- |
| Sync reboot |

Then, use ftrace …

First

mount -t tracefs nodev /sys/kernel/tracing

See the traces type activated by default :

function = function call

function_grap = function call and its sub-functions calls

nop

cat /sys/kernel/tracing/available_tracers

Visualize function call in graph

echo function_graph > /sys/kernel/tracing/current_tracer cat /sys/kernel/tracing/current_tracer function_graph

Start f traces

echo 1 > /sys/kernel/tracing/tracing_on

See all possible functions that can be traced

cat /sys/kernel/tracing/available_filter_functions

Filter the functions with i2c in the name (can be usb_, uart, etc)

echo "*i2c*" > /sys/kernel/tracing/set_graph_function

See all possible functions that can be traced: only i2c ones

cat /sys/kernel/tracing/available_filter_functions

Clear the uart selection (ic2 ones)

echo > /sys/kernel/tracing/available_filter_functions

Show the 20 first trace lines

cat /sys/kernel/tracing/trace ¦ head -20

Clean the trace

echo > /sys/kernel/tracing/trace

Example to show I2c interaction with call graph

echo function_graph > /sys/kernel/tracing/current_tracer echo "*i2c*" > /sys/kernel/tracing/set_graph_function echo 1 > /sys/kernel/tracing/tracing_on echo > /sys/kernel/tracing/trace cat /sys/kernel/tracing/trace i2cdetect –l cat /sys/bus/iio/devices/iio\:device2/in_temp_raw cat /sys/kernel/tracing/trace

I2cdetect activity kernel activity on target consol

Weston compositor call to Drm

echo "*drm*" > /sys/kernel/tracing/set_graph_function Weston-terminal

Weston traces log (on the target console)

It is possible show all the functions called without call graph.

This can be a first approach to check if the wanted function is effectively called.

Also we will catch more functions for a given trace depth size.

Then we can use the function call graph to see which function calls the “wanted” one.

Example to show all functions called whose name contains “i2c”

echo function > /sys/kernel/tracing/current_tracer cat /sys/kernel/tracing/current_tracer echo "*i2c*" > /sys/kernel/tracing/set_ftrace_filter echo 1 > /sys/kernel/tracing/tracing_on echo > /sys/kernel/tracing/trace cat /sys/kernel/tracing/trace i2cdetect –l cat /sys/kernel/tracing/trace

See the lab5 of “STM32MP1 hands on Trace and Debug.pptx” in here

You can find exercise on

  - Strace

  - Ftrace

  - LTTng

  - Perf - Perf top

  - FlameGraph

- Monitoring tool

Ethtool: usefull to get the Ethernet I/F state, configuration

iPerf3 : (not in list below) to test end-to-end Ethernet connection performances

Ifconfig :IP I/f

NetData :all system resources monitoring

- A7 step-by-step debugging with GDB (Gnu DeBugger)

Reference:

https://wiki.st.com/stm32mpu/wiki/GDB

With GDB, we can control the Debug cell IP inside the Soc.

Gdb is connected to GDB server (inside OpenOCD) that controls Jtag connection or ST-Link “probe”.

We can step-by-step debug the TF-A, U-boot, Kernel, Linux applications, M4 fw with ST-Link.

In the developer package, TF-A, Uboot, Kernel are compiled by default with the debugging symbols (CFLAGS += -g)

  - Debug TF-A

TF-A is running in A7 secure hw execution context.

- Get Setup.gdb for above page, Path_env.gdb and copie in your VM them into $HOME/gdbscripts

- Edit  Setup.gdb change it  to tell if you debug TF-A,Uboot,Kernel

Example for TF-A :

set $debug_phase = 1 <-debug TF-A fw

set $debug_mode = 0 <- debug from reset

set $debug_trusted_bootchain = 0

debug_mode =0 , GDB will reset the Soc and reboot. Debug_mode=1  GDB “attach” ie connect an already running code on target. It is not possible for TF-A.

- Update path in Path_env.gdb with your environment

get it from session5 repository here for development package environment

Example here for TF-A :

update “directory” with path to  tf-a

update the path of “symbol_bl2”, “symbol_bl32” with the path in your environement to  tf-a-bl2-trusted.elf 
and tf-a-bl32-trusted.elf

- Start OpenOCD server first (cf §5.2.4) (connected Soc Debug cell IP via the ST-Link)

Pre-requisite:  Your target is connected to ST-Link and is ON, Linux is running.

Open new terminal en linux host

cd /local/STM32MP15-Ecosystem-v1.0.0/Developer-Package/SDK source ./environment-setup-cortexa7t2hf-neon-vfpv4-openstlinux_weston-linux-gnueabi $OECORE_NATIVE_SYSROOT/usr/bin/openocd -s $OECORE_NATIVE_SYSROOT/usr/share/openocd/scripts -f ./sysroots/x86_64-openstlinux_weston_sdk-linux/usr/share/openocd/scripts/board/stm32mp15x_dk2.cfg

if you have issue to start OpenOCD:

Check that ST-Link is connected in Player/removable device menu.

Check that USB ST-Link drv is installed in your linux host ( stsw-link007, see https://wiki.st.com/stm32mpu/wiki/ST-LINK)

- start GDB (cf §5.2.4) to debug TF-A from boot

Pre-requisite:  Your target is connected to ST-Link and is ON, Linux is running.

Open new terminal en linux host

cd $HOME/gdbscripts/ source /local/STM32MP15-Ecosystem-v1.0.0/Developer-Package/SDK/environment-setup-cortexa7t2hf-neon-vfpv4-openstlinu x_weston-linux-gnueabi $OECORE_NATIVE_SYSROOT/usr/bin/arm-openstlinux_weston-linux-gnueabi/arm-openstlinux_weston-linux-gnueabi-gdb -x=$HOME/gdbscripts/Setup.gdb

GDB shows A7 is stopped at the beginning of “bl2_entrypoint()”

For all the command look in wiki at  https://wiki.st.com/stm32mpu/index.php/GDB_commands

GDB shows A7 is stopped in line 17 in bl2_entrypoint() , you can see the code :

list (+ enter to see more code)

See breakpoints  list: info breakpoint

Set breakpoints , delete breakpoint:  hb bl2_main    delete break 7

Continue execution to next breakpoint: cont

See the call stack with back trace: bt

Exit gdb : q

To access to variables, recompile TF-A with -o0 option to deactivate the gcc optimizations

  - Debug the Linux kernel

- Edit $HOME/gdbscripts/Setup.gdb

set $debug_phase = 4 <- debug kernel

set $debug_mode = 0

set $debug_trusted_bootchain = 0

- Edit path of Vmlinux in $HOME/gdbscript/Path_env.gdb

get it from session5 repository here for development package environment

- Open new terminal start OpenOcd

cd /local/STM32MP15-Ecosystem-v1.0.0/Developer-Package/SDK source ./environment-setup-cortexa7t2hf-neon-vfpv4-openstlinux_weston-linux-gnueabi $OECORE_NATIVE_SYSROOT/usr/bin/openocd -s $OECORE_NATIVE_SYSROOT/usr/share/openocd/scripts -f ./sysroots/x86_64-openstlinux_weston_sdk-linux/usr/share/openocd/scripts/board/stm32mp15x_dk2.cfg

- In a new terminal start GDB

cd $HOME/gdbscripts/ source /local/STM32MP15-Ecosystem-v1.0.0/Developer-Package/SDK/environment-setup-cortexa7t2hf-neon-vfpv4-openstlinux_weston-linux-gnueabi $OECORE_NATIVE_SYSROOT/usr/bin/arm-openstlinux_weston-linux-gnueabi/arm-openstlinux_weston-linux-gnueabi-gdb -x=$HOME/gdbscripts/Setup.gdb

Warning : press c to continue or enter

Set break point into  i2cdev_open : hb i2cdev_open

Let run the kernel  : cont

On the target, the I2cdetect application will call “i2cdev_open” function on which we have just set a breakpoint

(we saw this with ftace below)

Open a terminal on the target :

minicom –D /dev/ttyACM0

i2cdetect -l

Gdb shows A7 is stopped in the i2cdev_open :

  - Debug U-boot

- Edit $HOME/gdbscripts/Setup.gdb

set $debug_phase = 3  <- debug U-boot

set $debug_mode = 0

set $debug_trusted_bootchain = 0

- Edit path of Vmlinux in $HOME/gdbscript/Path_env.gdb

get it from session5 repository here for development package environment

- Open new terminal, start OpenOcd

cd /local/STM32MP15-Ecosystem-v1.0.0/Developer-Package/SDK source ./environment-setup-cortexa7t2hf-neon-vfpv4-openstlinux_weston-linux-gnueabi $OECORE_NATIVE_SYSROOT/usr/bin/openocd -s $OECORE_NATIVE_SYSROOT/usr/share/openocd/scripts -f ./sysroots/x86_64-openstlinux_weston_sdk-linux/usr/share/openocd/scripts/board/stm32mp15x_dk2.cfg

- In a new terminal, start GDB

cd $HOME/gdbscripts/ source /local/STM32MP15-Ecosystem-v1.0.0/Developer-Package/SDK/environment-setup-cortexa7hf-neon-vfpv4-openstlinux_weston-linux-gnueabi $OECORE_NATIVE_SYSROOT/usr/bin/arm-openstlinux_weston-linux-gnueabi/arm-openstlinux_weston-linux-gnueabi-gdb -x=$HOME/gdbscripts/Setup.gdb

Gdb shows A7 stopped entry point of U-boot

  - Debug an application

See the doc Lab-DeveloperPackageVx.docx in here

“Step9 : STM32MP1 Developer Package Debug Linux application with gdb”

See:

https://wiki.st.com/stm32mpu/wiki/GDB#Debug Linux application with gdbserver