Flashing Tool -Examples

April 2, 2018

- Purpose

- methods on how to flash the board

  - Get Starter Package from wiki

  - Flash with CubeProgrammer

    - Flash layout overview

    - Flash sdcard under windows

    - CubeProgrammer installation on Linux

    - Flash scard under Linux

2 Fast sdcard  cloning from a “raw” image

2.1 Create raw image

2.2 Flash Raw image on linux

2.3 Flash Raw image on windows

1.0 Get the starter package SDcard binary

Get starter package from

https://wiki.st.com/stm32mpu/index.php/STM32MP15_Discovery_kits_-_Starter_Package

for board revision C only (otherwise go to archive images

FLASH-stm32mp1-openstlinux-4.19-thud-mp1-19-02-20.tar.xz  (temporary repository)

https://wiki.st.com/stm32mpu/index.php/STM32MP1_Starter_Package_-_images

(soon available)

1.1 STM32CubeProgrammer flash-layout overview

https://wiki.st.com/stm32mpu/index.php/STM32MP1_Distribution_Package

- In Distribution package, the st-image-weston binaries to flash are under

$DISTRI_ROOT/$DISTRI_RELEASE/build-openstlinuxweston-stm32mp1/tmpglibc/deploy/images/stm32mp1

- Contains all binaries : partitions (bootfs,rootfs,…) + firmware (tf-a,Uboot,Uimage) + device tree (*.dtb)

- Under sub-directory flashlayout_st-image-weston are the  Flash layout descriptor *.tsv files

Different *.tsv depending on

  - Board (discovery , eval,),

  - booting devices (eMMC,NAND,SDCard,…)

  - boot chains (basic or trusted, op-tee)

- Flash layout descriptor *.tsv files list the partitions to flash with address and boot device.

*.tsv file is an Input of the STM32CubeProgrammer

Example FlashLayout_sdcard_stm32mp157c-dk2-trusted.tsv

1.2 Flash image on SDcard with STM32CubeProgrammer from windows

- Install STM32CubeProgrammer windows version

Open in web browser:

https://wiki.st.com/stm32mpu/index.php/STM32CubeProgrammer

download from internal repository and run installer (.exe)

  - Discovery switch to activate USB DFU bootloader

When programming the discovery board boot pins set to ‘00’ (off)

Connect the second USB type C cable to the board

USB ST-link cable for hyper terminal

Boot pins switch “near” “12”

- bootloader is in USB DFU mode

USB OTG cable for CubeProgrammer

USB Power cable

Check dfu driver is installed on windows

    - If you are using Win10 you may need

Download Zadig : http://zadig.akeo.ie/downloads/zadig-2.3.exe

Launch  Zadig.

Plug your board or just Reset it.

in Zadig : options-> List All Devices.

Choose your board from the device list (DFU ...)

Press Install Driver Button

- Run CubeProgammer under windows

Select USB

and the configuration shall be detected (SB1, serial number)

Then

Connect

You see the SDCard partitions

Open the FlashLayout_sdcard_stmp32mp157c-dk2-trusted.tsv file 
in “Flashlayout_st-image-weston” folder

Browse to set the binary path

binaries are in same folder level

as “Flashlayout_st-image-weston”

Then download

1.3 STM32CubeProgrammer installation on linux host

- Get STM32CubeProgrammer

Open in Linux web browser:

https://wiki.st.com/stm32mpu/index.php/STM32CubeProgrammer

For the moment use the internal repository:

On Linux host download, unzip SetupSTM32CubeProgrammer.zip on under ~/Desktop

Note: On VMware Linux host You can download from windows then drag&drop under the Desktop

Summary installation of the sections 2.1 and 2.2  of  https://wiki.st.com/stm32mpu/index.php/STM32CubeProgrammer

- Install STM32CubeProgrammer

cd ~/Desktop unzip "SetupSTM32CubeProgrammer v2.0.0.zip" -d "./SetupSTM32CubeProgrammer v2.0.0" cd ~/Desktop/SetupSTM32CubeProgrammer\ v2.0.0/ ./SetupSTM32CubeProgrammer-xxx.linux

Wizard installer is then started, let the default path/settings

sudo ln -s HOME/STMicroelectronics/STM32Cube/STM32CubeProgrammer/bin/STM32_Programmer_CLI $HOME/bin/STM32_Programmer_CLI $

  - Install DFU USB driver for USB OTG cable

sudo apt-get install libusb-1.0-0 sudo cp $HOME/STMicroelectronics/STM32Cube/STM32CubeProgrammer/Drivers/rules/*.* /etc/udev/rules.d/

  - Install Java if not already done (you will get a warning at gfx launch) 
before launching graphical interface or command line

(/home/osboxes/STMicroelectronics/STM32Cube/STM32CubeProgrammer/bin/STM32CubeProgrammer)

sudo apt-get install openjfx

1.4 Flash image on SDcard with STM32CubeProgrammer from Linux host

  - Flash & Run

  - Discovery switch to activate USB DFU bootloader

When programming the discovery board boot pins set to ‘00’ (off)

- Connect second USB type C OTG port is to Linux Host

- on VM before running commands,  activate the USB DFU driver use VM Player menu "removable devices" to connect to STMicroelectronics DFU in HS mode

Flashing the “st-image-weston“ image with SDcard partitions on a disco SDK2 with the trusted boot chain

for distribution package export DISTRI_BUILD_PATH=$DISTRI_ROOT/$DISTRI_RELEASE/build-openstlinuxweston-stm32mp1 export FLASH_IMAGE_DIR=$DISTRI_BUILD_PATH/tmp-glibc/deploy/images/stm32mp1 for starter package export FLASH_IMAGE_DIR=/<starterPackage>/../images/stm32mp1 export FLASH_IMAGE_TRUSTED=flashlayout_st-image-weston/FlashLayout_sdcard_stm32mp157c-dk2-trusted.tsv cd $FLASH_IMAGE_DIR STM32_Programmer_CLI -c port=usb1 -w $FLASH_IMAGE_TRUSTED

  - Set switch to boot from SDcard

    - When booting the discovery board boot pins set to ‘11’ (on)

- Flash raw image

  - Create raw image for sdcard

Install starter package on Linux machine

Create directory /local/STM32MP15-Ecosystem-v1.0.0/Starter-Package

mkdir /local

chmod 7777 /local

mkdir /local/STM32MP15-Ecosystem-v1.0.0

mkdir /local/STM32MP15-Ecosystem-v1.0.0/Starter-Package

starter_package_path  = /local/STM32MP15-Ecosystem-v1.0.0/Starter-Package/stm32mp1-openstlinux-4.19-thud-mp1-19-02-20/

Unzip starter package images

cd /local/STM32MP15-Ecosystem-v1.0.0/Starter-Package

tar -xvf FLASH-stm32mp1-openstlinux-4.19-thud-mp1-19-02-20.tar.xz

from starter package

<starter_package_path>/images/stm32mp1/scripts

/local/STM32MP15-Ecosystem-v1.0.0/Starter-Package/stm32mp1-openstlinux-4.19-thud-mp1-19-02-20/images/stm32mp1/scripts

or in distribution build

<distri_package_path>/build-openstlinuxweston-stm32mp1-boards-revb/tmp-glibc/deploy/images/stm32mp1-boards-revb/script

Run the script create_sdcard_from_flashlayout.sh

sudo ./create_sdcard_from_flashlayout.sh ../flashlayout_st-image-weston/FlashLayout_sdcard_stm32mp157c-dk2-trusted.tsv

The .raw file image is created under <starter_package_path>/images/stm32mp1/

See also

https://wiki.st.com/stm32mpu/index.php/How_to_populate_a_microSD_card_with_a_script

Mass market raw image for disco revC is shared under: \\PRGCWD0579.prg.st.com\TOMAS_Share\MPU\STM32MP1_STFAE_Part2\stm32mp1-session2-Linux introduction & tool set-up\ FlashLayout_sdcard_stm32mp157c-dk2-trusted.raw.zip

2.1 Flash raw image on sdcard with windows tools (fastest method)

The raw image contains all the partitions for SD card boot device.

Look on the wiki to build a raw image from the partitions.

FlashLayout_sdcard_stm32mp157c-dk2-trusted.raw for the discovery board revC

On window10 method

Use Etcher (for raspberry pie, download it from web)

1/ Plug your sdcard directly to your PC

2/ Get the raw image: FlashLayout_sdcard_stm32mp157c-dk2-trusted.raw

(shared on \\PRGCWD0579.prg.st.com\TOMAS_Share\MPU\STM32MP1_STFAE_Part2\stm32mp1-session2-Linux introduction & tool set-up)

3/Use Etcher to write the .raw image file on the scdcard, it will ask to format the scd card in fat32, no issue.

Window 7 method

Plug a external USB sdcard reader in windows PC

Install Ext2Fsd-0.69.exe

Then with Ext2Fsdand assign the sdcard drive to a letter for instance f:\

Win32diskmanager tool

Unzip win32diskimager-v0.9-binary.zip

Launch Win32DiskImager.exe and select sdcard drive (for instance f:\) as device

And select the raw image file

Then Write

Then Yes

Then sdcard is programmed with the image

2.2Flash raw image on SDcard from linux host (fast flashing method)

Look for article in wiki search: [How to populate a microSD card with a script]

The raw image contains all the partitions for SD card boot device.

Look on the wiki to build a raw image from the partitions.

1/ Plug on Linux host a USB SDcard reader (with the SDcard inserted)

2/ follow:

export FLASH_IMAGE_DIR= /local/STM32MP15-Ecosystem-v1.0.0/Starter-Package/stm32mp1-openstlinux-4.19-thud-mp1-19-02-20/images/stm32mp1 cd $FLASH_IMAGE_DIR export RAW_IMAGE=flashlayout_st-image-weston_FlashLayout_sdcard_stm32mp157c-dk2-trusted.raw lsblk Example in sdc umount /media/osboxes/rootfs umount /media/osboxes/userfs umount /media/osboxes/bootfs1 sudo dd if=$RAW_IMAGE of=/dev/sdc bs=1M conv=fsync status=progress