Tools - Setup

- Purpose

1 Check the board is booting and connect an hyper terminal

2 How to copy from Linux PC host to board target

2.1 With usb key

2.2 Board as usb mass storage of PC

2.3 copy via Ethernet over USB connection

3 How to install AC6 system workbench on Linux PC host

3.1 Install system workbench

3.2 Install CoprocMPU eclipse plug-in

4 How to install STM32CubeMx on Linux PC host

Color convention : in pink rectangle for command on the board

Color convention : in blue rectangle for command on Linux Host

1 Boot the board and connect an hyper terminal

All info about the board is there

(there is no user guide document as we have in the MCUs)

http://intranet.lme.st.com:8000/php-bin/ug_mcdmpu/index.php/STM32MP157X-DKX_-_hardware_description

USB OTG port to HOST PC

For CubeProgrammer

For usb over Ethernet

For Uboot in mass storage

Boot on SDcard (11)

Purpose: Interact with Discovery from a Linux Host hyper terminal window.

You can execute on target some linux commands you have entered into the hyper terminal window

Check connections with ST-Link

  - Ensure your SDCard is plugged

  - Connect the Linux host PC to the ST-LINK-v2 port of the discovery (Micro-USB cable)

  - Power-On the discovery with USB (USB C cable - power supply)

  - Install on your Linux host PC the hyper terminal application called “minicom”

apt-get install minicom

  - Enable the ST-Link USB connection to the Linux virtual machine

You should see the ST-Link usb connection on the list, select connect

  - Launch Hyper terminal on your Linux PC . The Uart is connection between the Linux PC and the discovery is done via the ST-LINK cable (2)

minicom -D /dev/ttyACM0

Default UART settings are OK , no need to change.

Baud rate 115200

Data 8bit

Parity none

Stop 1bit

Flow control none

  - Reset the board with black reset button,

then observe the log in the Hyper Terminal window on your Linux PC host :

The board is booted when you get the linux prompt

2 How to copy from Linux PC host to board target

We present 3 different methods to copy files to the target.

2.1 Copy to board using a USB key

Purpose:

you have a file on your USB Key (an JPEG image for ex) and you want to copy it on the target.

Insert usb key in board

Observe logs as follows

With the mount command USB key content is then visible under /mnt

mount –t vfat /dev/sda1 /mnt -v

To see the content:

ls /mnt

example of copy (need to use sync command to force the copy)

cp /mnt/<filename> /home/root cp –a /mnt/<directory_name> /home/root

sync

Step 2.2 : Copy to board seen as mass storage for Linux Host

(Linux host is connected to USB OTG board port)

Purpose:

you have a file on your Linux host (a binary just compiled for ex)

and you want to copy it on the target.

http://intranet.lme.st.com:8000/php-bin/ug_mcdmpu/index.php/How_to_use_USB_mass_storage_in_U-Boot

  - USB Plug cable on the USB OTG port

  - To enter in uboot shell : press Reset button on the target and during boot time Press any key in hyper terminal window in Linux host

  - On target in uboot prompt launch

ums 0 mmc 0

(The HyperTerminal is blocked)

On linux host rootfs and bootfs partition are available in /media/<user>/bootfs where "<user>" is the user account.

In case rootfs or bootfs are not mounted from linux, click on launcher bar (new usb folder icons)

On Linux host open a new console call the following commands to copy in target “/” directory

cp {path on linux host}/your file /media/user/home/rootfs then unmount usb drive

For example to copy stm32_usbo_eth_config_new.sh from your Linux host desktop

into your target home directory (~ or /home/root)

cp /home/Desktop/stm32_usbo_eth_config.sh /media/<user>/rootfs/home/root/ then unmount usb drive

2.3 Copy via Ethernet over USB connection

(Linux host is connected to USB OTG board port)

Purpose:

You have a file on your Linux host (a binary just compiled for ex)

and you want to copy it on the target.

We make an Ethernet connection using the USB OTG cable (to avoids the Ethernet cable and router)

We use the linux USB Ethernet gadget.

2.3.1 Discover your environment (only once)

  - Connection

connect the USB OTG type C cable to the PC

USB OTG port is the white cable

  - Observe the name of the Ethernet over USB i/f on Linux host of the USB OTG cable

Get the name of the Ethernet over USB i/f on Linux Host of the USB OTG cable

On host, observe the list all the Ethernet i/f:

ifconfig

seek for the name of i/f that correspond to  Ethernet USB i/f

here we see : " ens35u1 Link encap:Ethernet  HWaddr 7e:cd:ed:45:72:72

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1"

To identify the name, disconnect the USB OTG cable

Re-type ifconfig, the name of the I/F disappears in the list given by ‘ifconfig’.

- On target, the IP address is 192.168.7.2 assigned by default

On the target in the hyper terminal window

in /home/root (or ~) directory

Ifconfig

when the OTG cable (typeC)  is connected

we see

"usb0 Link encap:Ethernet  HWaddr ….

inet addr:192.168.7.2  Bcast:

2.3.2 Configure Linux host Ethernet over USB i/f

Once you know the name of Linux host Ethernet over USB i/f of the USB OTG cable.

Connect the USB OTG cable

If Linux host starts a DHCP address search, network top right icon there show a "searching pictogram"

  - Click on top right network icon and click on "Disconnect" of the "Wired connection2"

=> A Ubuntu Pop-up "Disconnected" should happen; Then the network icon show vertical arrows

  - Assign 192.168.7.1 address to Linux host Ethernet USB i/f

Before you know, the name of your Linux host Ethernet USB i/f (see above)

here it is : “ens35u1”

Assign to this i/f the 192.168.7.1  address with the command :

sudo ifconfig ens35u1 192.168.7.1 netmask 255.255.255.0

To check  the Linux Host :

Ifconfig ens35u1 (If not up sudo ifup ens35u1)

you should see  :"ens35u1   Link encap:Ethernet  HWaddr 4a:83:81:8b:a1:3b

inet addr:192.168.7.1  Bcast:192.168.7.255  Mask:255.255.255.0

inet6 addr: fe80::4883:81ff:fe8b:a13b/64 Scope:Link

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

…."

If the connection is shut-down in VMWare

=> reassign the Ip :

sudo ifconfig ens35u1 192.168.7.1 netmask 255.255.255.0

2.3.3 Check Ethernet over USB connection

  - Check connection :

192.168.7.2 (address of the target)

192.168.7.1 (address of the Linux host)

On Linux Host

ping 192.168.7.2

On board

ping 192.168.7.1

    - Configure Linux host Ethernet over USB i/f permanently

If the connection name is always the same after each cable connection,

You can setup permanently the connection as follows

- Assign an IP address to Linux host (192.168.7.1) to your Ethernet USB i/f name (here "ens35u1 “)

In Linux host edit as follow

sudo gedit /etc/network/interfaces &

Add the Ethernet i/f name you have (here ens32u1)

# Ethernet/RNDIS gadget (g_ether)

allow-hotplug ens35u1

iface ens35u1 inet static

address 192.168.7.1

netmask 255.255.255.0

network 192.168.7.0

gateway 192.168.7.1

- Disable the networkManager for the all the Ethernet USB i/f starting by ens35* (=name of your i/f)

-Otherwise it searchs for DHCP IP address during 30s then any IP previously assigned to the I/f is lost

- In linux host edit as follow :

sudo gedit /etc/NetworkManager/conf.d/stm32mp-otg-eth.conf &

Add in file

[keyfile]

unmanaged-devices=interface-name:ens35*;interface-name:usb*

- Reboot the Linux Host

    - Copy via Ethernet over USB

Once the Ethernet Over USB link is well configured and up,

you can use the scp command to make copies between the target and the Linux host machine.

scp -r {path}/* root@<ip address>:/

On target, force the file system to update the copy into the sdcard

by

Sync

For example,

If you have a folder on Linux host machine under /opt/tm32mp1-3day/01 STM32MP1_Getstarted

that you want to copy on the target root directory “/”

make the command on Linux Host:

scp -r /opt/tm32mp1-3day/01 STM32MP1_Getstarted/ root@192.168.7.2:/

Sync

3 Installation of STM32Cube_MPU Package

https://wiki.st.com/stm32mpu/index.php/STM32-CoPro-MPU_plugin_release_note

  - Installation of  SystemWorkbench 2.8.0 +  STM32-CoPro-MPUMicroprocessor Unit plugin version 1.0.0

Package with eclipse Neon + st-link + st openOCD + Coproc plug-in

- Ensure SystemWorkbench 2.6 is not already installed on your Linux host

Otherwise in folder where eclipse binary is stored  run : java -jar uninstall.jar

- Download SystemWorkbench 2.8 installer for Linux 64 bits

https://wiki.st.com/stm32mpu/index.php/STM32-CoPro-MPU_plugin_release_note

you need to be logged in the wiki to see internal repository

Or for customers

    - Register or login www.openstm23.org

    - From page

http://www.openstm32.org/Downloading%2Bthe%2BSystem%2BWorkbench%2Bfor%2BSTM32%2Binstaller

copy on Linux host ~   install_sw4stm32_linux_64bits-v2.8.run

- Open Terminal

cd ~/Desktop

./install_sw4stm32_linux_64bits-v2.8.run (in manual mode, answer yes to all questions)

- Run click on "SystemWorkbench for STM32" icon on Desktop 
or /Ac6/SystemWorkbench/eclipse&

The first time Systemworkbench is started , the gcc tool chain is installed.

To check release install :

eclipse menu  “Help” -> “installation details” search for

" OpenSTM32IDE        2.8.0. …..        fr.ac6.feature.mcu.ide.feature.group        Ac6"

Check release install : Help / "installation details" menu  search for

“STM32-CoPro-MPU  -  Cortex-M debug support on Hybrid architecture SoC
        1.0.0. …..        com.st.stm32ide.mpu.feature.debug.feature.group        STMicroelectronics”

4 Installation of STM32CubeMX

https://wiki.st.com/stm32mpu/index.php/STM32CubeMX

Get CubeMx version from

https://www.st.com/en/development-tools/stm32cubemx.html

mkdir ~/STM32MPU-Tools

cd ~/STM32MPU-Tools/SetupSTM32CubeMX-xxx Unzip SetupSTM32CubeMX-xxx.zip

Open in web browser  ~/STM32MPU-Tools/SetupSTM32CubeMX-xxx/Readme.html

Check Install Java JRE 1.7.0_45 or a more recent version:

sudo apt-get install default-jre ./SetupSTM32CubeMX-xxx.linux

Launch:

| ~/STM32CubeMX/STM32CubeMX & |  |
| --- | --- |